Experiment: 5

Simulation and Throughput Analysis of LTE-Advanced Carrier Aggregation

Objective – 1: Model One Component Carrier

Define a single LTE-Advanced component carrier and estimate its throughput for a selected

bandwidth and spectral efficiency.

MATLAB Program

clc; clear; close all;

BW1 = 20; % Component carrier bandwidth (MHz)

SE = 4; % Spectral efficiency (bits/s/Hz)

Ncc = 1; % Number of component carriers

BWagg = BW1*Ncc;

Throughput = BWagg*SE; % Mbps for MHz × bits/s/Hz

disp('Single Component Carrier')

disp(['Bandwidth = ',num2str(BWagg),' MHz'])

disp(['Estimated Throughput = ',num2str(Throughput),' Mbps'])

figure

bar(Ncc,Throughput)

grid on

xlabel('Number of Component Carriers')

ylabel('Estimated Throughput (Mbps)')

title('LTE-Advanced Single Carrier Throughput')

Single Component Carrier
Bandwidth = 20 MHz
Estimated Throughput = 80 Mbps

Objective – 2: Extend to Multiple Aggregated Component Carriers

Aggregate multiple component carriers and determine the resulting total bandwidth and

estimated throughput.

L

MATLAB Program

clc; clear; close all;

BWcc = 20; % MHz per component carrier

SE = 4; % bits/s/Hz

CC = [1 2 3]; % Number of component carriers

AggBW = BWcc .* CC; % MHz

Throughput = AggBW .* SE; % Mbps

disp('CC Aggregated BW (MHz) Estimated Throughput (Mbps)')

disp([CC' AggBW' Throughput'])

figure

bar(CC,Throughput)

grid on

xlabel('Number of Aggregated Component Carriers')

ylabel('Estimated Throughput (Mbps)')

title('LTE-Advanced Carrier Aggregation Throughput')

CC Aggregated BW (MHz) Estimated Throughput (Mbps)
     1    20    80
     2    40   160
     3    60   240

Objective – 3: Compare Estimated Throughput for Different Carrier Bandwidths

MATLAB Program

clc; clear; close all;

BW = [5 10 15 20]; % Carrier bandwidths (MHz)

SE = 4; % bits/s/Hz

TP = BW .* SE; % Estimated throughput (Mbps)

disp('Bandwidth (MHz) Throughput (Mbps)')

disp([BW' TP'])

figure

plot(BW,TP,'o-','LineWidth',2)

grid on

xlabel('Carrier Bandwidth (MHz)')

ylabel('Estimated Throughput (Mbps)')

title('Throughput vs Component Carrier Bandwidth')

Expected Output: Throughput increases as the component-carrier bandwidth increases.

Objective – 4: Explain the Effect of Carrier Aggregation

The total available bandwidth is obtained by combining the component carriers. Under the

simplified constant spectral-efficiency model, throughput increases approximately in

proportion to the aggregated bandwidth.

Result

1. The LTE-Advanced link was successfully modeled using a single component carrier.

2. Multiple component carriers were aggregated and the resulting bandwidth and estimated

throughput was calculated.

3. Throughput was compared for different carrier bandwidths.

4. The simulation demonstrated that carrier aggregation increases available transmission

bandwidth and can increase achievable throughput.