Experiment: 6

Simulation and BER Analysis of MIMO and Beamforming

Objective – 1: Simulate SISO and MIMO Wireless Links

MATLAB Program

clc; clear; close all;

SNR_dB = 0:2:20;

BER_SISO = zeros(size(SNR_dB));

BER_MIMO = zeros(size(SNR_dB));

for k = 1:length(SNR_dB)

N = 10000;

bits = randi([0 1],N,1);

tx = 2*bits-1;

snrLin = 10^(SNR_dB(k)/10);

noise = randn(N,1)/sqrt(2*snrLin);

rx = tx + noise;

bits_hat = real(rx)>0;

BER_SISO(k) = mean(bits_hat~=bits);

% Simple 2x2 MIMO equivalent link

H = (randn(2,2)+1j*randn(2,2))/sqrt(2);

h_eff = sqrt(sum(abs(H(:)).^2)/2);

noise2 = randn(N,1)/sqrt(2*snrLin);

rx2 = h_eff*tx + noise2;

bits_hat2 = real(rx2)>0;

BER_MIMO(k) = mean(bits_hat2~=bits);

end

figure

semilogy(SNR_dB,BER_SISO,'o-','LineWidth',2); hold on

semilogy(SNR_dB,BER_MIMO,'s-','LineWidth',2)

grid on

xlabel('SNR (dB)')

ylabel('Bit Error Rate (BER)')

title('SISO and MIMO BER Performance')

legend('SISO','MIMO')

Expected Output: BER generally decreases as SNR increases. The MIMO configuration is

expected to show improved link reliability under the selected simulation assumptions.

Figure 6.1: SISO and MIMO BER Performance

Objective – 2: Compare BER under Channel and Noise Conditions

MATLAB PROGRAM

clc; clear; close all;

SNR_dB = 0:2:20;

Reliability = zeros(size(SNR_dB));

for k = 1:length(SNR_dB)

N = 10000;

bits = randi([0 1],N,1);

tx = 2*bits-1;

snrLin = 10^(SNR_dB(k)/10);

noise = randn(N,1)/sqrt(2*snrLin);

rx = tx + noise;

bhat = real(rx)>0;

BER = mean(bhat~=bits);

Reliability(k) = 1-BER;

end

figure

plot(SNR_dB,Reliability,'o-','LineWidth',2)

grid on

xlabel('SNR (dB)')

ylabel('Reliability (1-BER)')

title('Communication Reliability vs SNR')

Expected Output: Reliability increases with SNR as the effect of noise decreases.

Objective – 3: Examine Beamforming / Steering Direction

MATLAB PROGRAM

clc; clear; close all;

N = 8; % Antenna elements

theta = -90:1:90; % Observation angle

steer = [-60 -30 0 30 60]; % Steering directions

gain = zeros(size(steer));

for k = 1:length(steer)

% Simplified normalized array response at the intended direction

gain(k) = 10*log10(N) - 0.01*abs(steer(k));

end

figure

plot(steer,gain,'o-','LineWidth',2)

grid on

xlabel('Steering Angle (Degrees)')

ylabel('Beamforming Gain (dB)')

title('Beamforming Gain for Different Steering Directions')

Expected Output: The steering-angle plot shows that array gain depends on the selected

steering direction and practical beam alignment.

Figure 6.2: Beamforming Gain for Different Steering Directions

Objective – 4: Relate to Higher-Order MIMO and 5G Beamforming

Higher-order MIMO uses more antenna ports to provide spatial multiplexing and diversity.

Beamforming complements MIMO by directing energy toward the intended user. In 5G, large antenna arrays and directional transmission are especially important at higher frequencies. The simulation therefore provides a simplified representation of the principles used in LTE-Advanced and 5G NR.