Experiment: 7

Analysis of 5G NR Frame Structure and Subcarrier Spacing

Objective – 1: Generate / Inspect a Simplified 5G NR Resource Grid

Program:

clc; clear; close all;

Nsc = 24; % Simplified number of subcarriers

Nsym = 14; % OFDM symbols per slot

grid = zeros(Nsym,Nsc);

% Simplified DL assignment: symbols 1-10

grid(1:10,:) = 1;

% Simplified UL assignment: symbols 11-14

grid(11:14,:) = 2;

figure

imagesc(grid)

grid on

xlabel('Subcarrier Index')

ylabel('OFDM Symbol Index')

title('Simplified 5G NR Time-Frequency Resource Grid')

colorbar

Expected Output: A time-frequency grid representing simplified downlink and uplink

resource assignments.

Objective – 2: Compare SCS and Slot Duration

Program:

clc; clear; close all;

scs = [15 30 60 120 240]; % kHz

mu = 0:4;

slot = [1 0.5 0.25 0.125 0.0625]; % ms

figure

plot(scs,slot,'o-','LineWidth',2)

grid on

xlabel('Subcarrier Spacing (kHz)')

ylabel('Slot Duration (ms)')

title('5G NR Slot Duration for Different Subcarrier Spacing')

figure

bar(scs)

grid on

set(gca,'XTickLabel',mu)

xlabel('Numerology Index (\mu)')

ylabel('Subcarrier Spacing (kHz)')

title('5G NR Subcarrier Spacing for Different Numerologies')

Expected Output: Slot duration decreases as SCS increases. The SCS graph shows 15, 30, 60,

120 and 240 kHz.

Figure 7.2: 5G NR Slot Duration for Different Subcarrier Spacing

Objective – 3: Identify Basic DL and UL Resource Assignments

Program:

clc; clear; close all;

symbols = 1:14;

DL = [1 1 1 1 1 1 1 1 1 1 0 0 0 0];

UL = [0 0 0 0 0 0 0 0 0 0 1 1 1 1];

figure

plot(symbols,DL,'o-','LineWidth',2); hold on

plot(symbols,UL,'s-','LineWidth',2)

grid on

xlabel('OFDM Symbol Index')

ylabel('Allocation Status')

title('Simplified Downlink and Uplink Assignment')

legend('Downlink','Uplink')

Expected Output: A plot identifying simplified DL and UL resource assignments across the

OFDM symbols.

Figure 7.3: Simplified Downlink and Uplink Assignment

Objective – 4: Relate Resources to gNodeB, RAN and RRC

The gNodeB is the principal 5G NR radio access node. It participates in radio-resource

management and scheduling and provides the radio interface between the UE and the 5G

core. RRC is a control-plane protocol responsible for radio-resource configuration and

connection-related control. The simplified grid therefore represents the radio resources that

are scheduled and configured within the RAN; the exact allocation in a live network depends

on traffic, channel conditions, QoS and scheduling decisions.

Result

1. A simplified 5G NR resource grid was generated and inspected.

2. The effect of different SCS values on slot duration was analyzed.

3. Basic downlink and uplink resource assignments were represented.

4. The relationship among NR radio resources, gNodeB operation, RAN organization and

RRC was explained.